NOTICE

Weight and balance or operating limitation changes shall be entered in the appropriate aircraft record. An alteration must be compatible with all previous alterations to assure continued conformity with the applicable airworthiness requirements.

8. Description of Work Accomplished (If more space is required, attach additional sheets. Identify with aircraft nationality and registration mark and date work completed.) Cessna xxx Nxxxxx Certification Basis CAR3 ### EFIS System - Installed Dynon EFIS- D10A in accordance with Dynon Avionics installation guide 100341-000 RevC and using FAA Document Non-approved EFIS installations in GA Aircraft as a guide. - Install magnetic heading and OAT temperature sensor at min-span left wing. - No primary flight, navigation or engine instruments required per CAR3.655 or CAR3.666 removed. - Electrical installation per AC43.13-1B Chapter 11. - Aircraft type design change from VFR/IFR to VFR day/night only. - VFR Day/Night placard installed. - ACO approved EFIS supplement to owner’s manual dated ___________ added to aircraft records. - Instructions for continued airworthiness are contained in Dynon Avionics installation guide 100341-000 RevC appendix A. Standard inspection times per Cessna 100 series Maintenance manual Section 2 - Equipment list and weight and balance amended. ************************************************************************************************ **** END ****

 Additional Sheets are Attached